Simultaneous operation of Double Rake Probe (DRP) and Peti Probe (PP) was carried out today.

The probe position and measurement regime fas following:

#33225 - #33228 – both probes behind the limiter, DRP in floating regime

lunch – DRP-R6 stopped measuring

#33232 – PP and DRP behind the limiter, DRP in I_sat

#33233 – no pre-ionization → no plasma

#33234 – the deepest DRP pin and PP Langmuir probe 75 mm from plasma center,

DRP in I_sat regime, interesting shot, strange data from DRP

http://golem.fjfi.cvut.cz/shots/33234/DASs/DoubleRake/das.html

#33235 – DRP and PP still the same position, DRP in floating.

#33236 – DRP 100 mm, PP 65 mm, DRP floating

#33237 – DRP and PP 65 mm, DRP floating

#33238 – DRP 100mm, PP 80 mm, DRP floating

#33239 – DRP 100mm, PP 80 mm, DRP floating

Depth of probe insertion is now part of dicharge command. Also the DRP regime is now part of it encoded in 12 character string, where ‘i’ is for I_sat, ‘f’ is for floating and ‘x’ for not connected.