Research Proposal for the 6th IAEA JOINT EXPERIMENT

From:	Gennadii Vorobjev St. Petersburg State University

Date:	15 October 2013

(Joint Experiment will be held on November 11 - 15, 2013)

Proposed Research Topic:

Investigation of RF pre-ionisation on GOLEM tokamak

Purposes:

To continue tests of low-power ECR preionization for plasma formation on GOLEM tokamak

Background:

Optimisation of the use of HTS PF coils on GOLEM requires modifications to the discharge scenario. To reduce AC losses during current ramp-up in HTS coils, reduction in the current ramp-up speed is needed, as this will result in a slower ramp-up of the current in coils. One of possible solutions may be reduction in the loop voltage needed for the plasma breakdown. It is well known that RF pre-ionisation is a powerful tool to achieve this goal [1].

Scope:

We propose to continue experiments with the low-power magnetron at the EC fundamental harmonics for the toroidal field of ~ 0.1T at 2.45GHz, ~1kW injected power with the aim to reduce requirements for the breakdown electric field.

To test the method of the vacuum vessel cleaning by means of the RF discharge.

Method:

- Experimental investigation of RF pre-ionisation on GOLEM tokamak. About 20-30 plasma pulses will be needed to optimise parameters. Can be done in one or two sessions, possibly in parallel with other experiments.

Needed equipment:

- The magnetron with all supporting structures, data acquisition for standard diagnostics.

- References:

- B Lloyd at al, NF 31 (1991) 2031